3 Fingers' Highest Visual Quality Autoexec.cfg for Voodoo II /Quake2

Version .95

7/14/98      Written by: Brett "3 Fingers" Jacobs    bjacobs@home.com

LOWER YOUR BRIGHTNESS SLIDER IN YOUR QUAKE2 VIDEO OPTIONS MENU A COUPLE OF NOTCHES AFTER INSTALLING THE AUTOEXEC.CFG

This autoexec.cfg was designed to give the absolute best image possible without regard to framerate loss, and is intended for high-end hardware users with Voodoo II 3D cards. The included autoexec.cfg will greatly enhance the visual quality of Quake 2 in all single player and multiplayer games. If you don't have a AMD K6-2 w/ 3D Now, Pentium II or Pentium Pro, I would not recommend using this autoexec.cfg for multiplayer games as you will get severe slowdown in battles when 4 or 5 players are in your field of view firing. Also this autoexec.cfg nearly eliminates the grainy texture problem that some Voodoo II users experience.

If you want to try it on a low end machine then change the gl_round_down to "1" in the autoexec.cfg and don't check any of the boxes in your Voodoo II control panel.

INSTRUCTIONS

Place the included autoexec.cfg in your Quake2/baseq2 folder and also place a copy of the autoexec.cfg in the Quake2/ctf folder if you play CTF. If you play LMCTF, also put a copy of the autoexec.cfg in the quake2/lmctf folder.

Next go to your Voodoo II control panel/advanced tab and place a check mark in the "force triplebuffering for glide apps" box. Also insure you DO NOT have a check in the box next to "Don't synch buffer swaps to monitor refresh rate for glide".    Then place a check mark in the box labled "force mipmap dithering for glide apps". The latest 3DFX Reference Drivers drivers have this "force mipmap dithering for glide apps" setting. It was called "enable trilinear filtering" (or something like that) for the RC2 drivers. Monster 3D II drivers may not have some of these check boxes. You can get the latest drivers on my webpage or here:    http://www.3dfx.com/download/download.html

If your Voodoo II control panel does not have a setting for "Force Mipmap Dithering For Glide Apps". This will cost some performance but will eliminate the banding you see in long hallways. You can enabled it in your Quake2 batch file or autoexec.bat using:

SET FX_GLIDE_LOD_DITHER=1

0=OFF

1=ON

If your Voodoo II control panel does not allow triple buffering to be enabled via the control panel, then add this line to your Quake2 batch file or put it in your autoexec.bat file. Reboot if you put it in your autoexec.bat.

SET FX_GLIDE_ALLOC_COLOR=3

NOTE: I have been getting reports that enabling triple buffering in the Voodoo II control panel or batch file can cause freezing gameplay and jerky movement for some users. If you experience this, uncheck the box in your Voodoo II control panel. You can leave the gl_triplebuffer 1 in the autoexec.cfg. The check box in the Voodoo II control panel or adding it to the batch file is the master over the gl_triplebuffer in the autoexec.cfg

NOTE: The cl_maxfps setting in the autoexec.cfg is set to "90" (default). If you have problems with high ping, phone jack icons, freezing gameplay and packet loss. Lower this setting down to the 20-37 range. This will cap your FPS and lower your bandwidth requirements.

IF YOU FEEL THE IMAGE IS TOO BRIGHT

If you feel the image is too bright with this autoexec.cfg after lowering your brightness slider in your Quake2 video options menu, lower the gl_modulate setting in the autoexec.cfg. It is currently set to 2.5, the default is 1.0. (Range is from 0.0 to 5.0) The higher the setting the less darkness there will be and will allow you to see players in dark areas that you normally could not.

BOOST YOUR FRAMES PER SECOND

The setting (set gl_round_down "0") in the autoexec.cfg will make the monster skin images and all the pick-up items like weapons, health and ammo packs etc. slightly more in focus. This setting of "0" takes quite a hit in FPS on the 8MB Voodoo II (about 20%) as it will cause you to run out of texture memory and swap textures. The 12mb Voodoo II has no problem running this setting of "0", but it is still slightly slower on the 12MB Voodoo II (1 or 2 fps). 8 MB Voodoo II users should change it to "1" in the autoexec.cfg if they feel they are losing to much performance.

TO RESTORE ALL THE DEFAULT SETTINGS

To restore all the default settings if you decide you do not like the autoexec.cfg. First remove the autoexec.cfg files and place the included "restore.cfg" in each folder that you had the autoexec.cfg and start every one of those games (single player, CTF, LM CTF etc.) and type at the console for each game:

exec restore.cfg <enter>

Then remove all the "restore.cfg" files and reinstall your old autoexec.cfg (if you used one) that you used prior to using this Hi Visual Quality autoexec.cfg.

Note: If at anytime you feel you cannot restore your settings to the default. Just delete the autoexec.cfg and the config.cfg from the quake2/baseq2 folder and the config.cfg and autoexec.cfg    files from any other folder you had the autoexec.cfg in. Then start a Quake2 game and Quake2 will generate a new config.cfg with all the default settings in place.

Best Regards,

Brett "3 Fingers" Jacobs

You Can Download The Latest Version Of "The 3 Fingers' Quake 2 Tweak Guide" At:

http://www.voodooextreme.com/3Fingers/

If you want the most comprehensive guide available on tweaking GL Quake and GL Quake World. Download the:

The 3 Fingers' Ultimate Tweak Guide For GL Quake & GL QW

http://www.voodooextreme.com/3Fingers/